# LensCoat® Nikon EDG VR Fieldscope 20-60x85 Spotting Scope (straight) INSTRUCTIONS

Piece 4

Remove the scope front element cover from the front of scope so sliding the pieces on will be easier and have the hood not extended. Carefully slide piece 4 cover into place just in front of the scope tripod collar.

Piece 3

Turn piece 3 (focus ring cover) inside out so that the nylon material is on the inside and the raw neoprene rubber is on the outside, this will help the piece slide down the scope. With the piece inside out carefully stretch and slide piece 3 down the scope to the focus ring. Then pull the cover over itself to make the right side out so the nylon material is on the outside.

Piece 2

Turn piece 2 inside out so that the nylon material is on the inside and the raw neoprene rubber is on the outside, this will help the piece slide down the scope. With the piece inside out have the narrow end point towards the front element carefully stretch and slide piece 2 down the scope to just after the focus ring. Then pull the cover over itself to make the right side out so the nylon material is on the outside.

Piece 1

You can now put the scope front element cover back on and carefully slide piece 1 into lining up the cut out rectangle with the front element cover hinge.

Piece 5

Slide piece 5 on the section behind the tripod collar lining up the hole for the scope knob.

Piece 6

Slide piece 6 on the eyepiece lining up the hole with the lock switch.

Piece 7

Slide piece 7 on the end of eyepiece.

Scope tripod collar

The LensCoat® Nikon EDG VR Fieldscope 20-60x85 Spotting Scope (straight) cover kit includes a strip of neoprene with adhesive tape applied to the back for the tripod collar.  Cutout the desired length/shape for the parts of the scope collar you want to cover.  Cut the longest piece first.  After all the cutting is done then remove the adhesive tape backing by carefully folding back a small corner and apply to the scope collar.

Velcro barrel piece

The LensCoat® Nikon EDG VR Fieldscope 20-60x85 Spotting Scope cover kit includes an optional piece that can be affixed to the scope via the Velcro strip ONLY when the scope hood is fully extended.

LensCoat.com®

80 Throckmorton St

Freehold, NJ 07728